Chapter Exam

Appendix C-The Basics of Logic Design

2013/03/05

- What are the differences between combinational logics and sequential logics? (10%)

- (a) Prove that the two equations E1, E2 are equivalent (10%)

E1 = (( A · B ) + (B · C )) · ( A · B · C )

E2 = ( A · B · C ) +  ( A · B · C )

(b) Try to use DeMorgan’s theorems to translate the E to product of sums form (10%)

E = ( A · B · C  ) + ( A · C · B ) + ( B · C · A ) + ( A · B · C )

- (a) Used 2-input NOR gate to implement :

- AND function. (5%)

- OR function. (5%)

(b) Implement the function F (A, B, C, D) = A · B + (~C · ~D) by :

    - 2-input AND and OR gates and inverters. (5%)

    - 2-input NAND gates. (10%)

- There are two inputs(A、B) and a selection signal(S).

The output C=A when S=0.

The output C=B when S=1.

Please write down the truth table, try to use Karnaugh map to reduce it and use sum-of-product form to represent it. Finally use gates to implement the circuit. (20%)

- Draw a 1-bit half adder in combination logic(OR、AND and NOT gates) with the input a, b and with the output sum, carry_out. (15%)

- Draw a 1-bit full adder in combination logic(XOR and AND and OR gates) with the input a, b, carry_in and with the output sum, carry_out by using 2 1-bit half adder (20%)