Chapter Exam

Appendix C-The Basics of Logic Design

2013/03/19

- What is the difference between Moore and Mealy machine? (5%)

- What is the difference between synchronous and asynchronous logic? (5%)

- Complete timing diagram according to CLOCK、D(input) signals separately.

- D latch : (10%)

- D flip-flop (rising edge) : (10%)

- Please use two NOR gates to implement S-R latch logic diagram (5%) and write down the function table (10%)

- According to the following diagram (JK flip-flop), please write down the function table when the clock is equal to 1 (15%) and compare the difference between SR latch and JK flip-flop(5%)

- Derive the logic equations (5.a, 5.b, 5.c, 5.d and 5.e) of a 16-bit CLA (carry look ahead) adder below. There is no need to simplify the equations. (25%)

| gi = ai · bi pi = ai + bi P0 = p3 · p2 · p1 · p0 P1 = p7 · p6 · p5 · p4 P2 = p11 · p10 · p9 · p8 P3 = 5.a G0 = g3 + (p3 · g2) + (p3 · p2 · g1) + (p3 · p2 · p1 · g0) G1 = g7 + (p7 · g6) + (p7 · p6 · g5) + (p7 · p6 · p5 · g4) G2 = 5.b G3 = 5.c C1 = G0 + (P0 · c0) C2 = G1 + (P1 · G0) + (P1 · P0 · c0) C3 = 5.d C4 = 5.e |  |
| --- | --- |

- A friend would like you to build an “electronic eye” for use as a fake security device. The device consists of three lights lined up in a row, controlled by the outputs Left, Middle, and Right, which, if asserted, indicate that a light should be on. Only one light is on at a time, and the light “moves” from left to right and then from right to left. Note that the rate of the eye’s movement will be controlled by the clock speed (which should not be too great) and that there are essentially no inputs.

- Draw the graphical representation for the finite-state machine used to specify the electronic eye.(10%)

- Assign state numbers to the states of the finite-state machine you constructed and write a set of logic equations for each of the outputs, including the next-state bits. (10%)