Chapter Exam

Chapter 2-instructions:language of the computer

2013/04/30

- This diagram is base addressing format of MIPS.

Please draw similar diagrams for the following MIPS addressing modes. (40%)

- Immediate addressing

- Register addressing

- PC-relative addressing

- Pseudo-direct addressing

- The following problem deal with translating from C to MIPS. Assume that the variables f, g, h, i, and j are assigned to registers $s0, $s1, $s2, $s3, and $s4, respectively. Assume that the base address of the arrays A and B are in registers $s6 and $s7, respectively

| a. | A[8] = g + A[i]; |
| --- | --- |
| b. | If (f < g) go to Less; |

For the C statements above, what is the corresponding MIPS assembly code?  ( You can use $t0~$t7 registers for temporal data. ) (30%)

- For the table, translate this ARM assembly code to MIPS assembly code. Assume that ARM register r0, r1, and r2 hold the same values as MIPS register $s0, $s1, and $s2, respectively. Use MIPS temporary register ($t0, etc.) where necessary. (30%)

| a. | MOV r0, #10 ; init loop counter to 10 LOOP: ADD r0, r1 ; add r1 to r0 SUBS r0, 1 ; decrement counter BNE LOOP ; if Z=0 repeat loop |
| --- | --- |
| b. | ROR r1, r2, #4 ; r1= r23:0 concatenated with r231:4 |

- What is the MIPS assembly code to load the following 32-bit constant into register $s0?

0000 0000 0011 1101 0000 1001 0000 0000  (10%)

- Please complete the translation of the following MIPS instructions to machine language (in decimal): (3 point for each) (30%)

Loop:  sll	$t2, $s3, 2

add	$t1, $t2, $s6

lw	$t0, 0($t1)

bne	$t0, $s5, Exit

addi	$s3, $s3, 1

addi	$t0, $t0, 5

j	Loop

Exit:

| (a) | 0 | 19 | 10 | 2 | 0 |
| --- | --- | --- | --- | --- | --- |
| 0 | (b) | (c) | (d) | 0 | 32 |
| 35 | (e) | (f) | 0 |  |  |
| 5 | (g) | (h) | (i) |  |  |
| 8 | 19 | 19 | 1 |  |  |
| 8 | 8 | 8 | 5 |  |  |
| 2 | (j) |  |  |  |  |
| ………………………… |  |  |  |  |  |

40020

40024

40028

40032

40036

40040

40044

40048

- Explain why most of today’s computer system use 2’s complement instead of sign magnitude (符號大小表示法)in their hardware implementations.(5%)

- Prove that “-x = x + 1” in 2’s complement representation. (5%)

- An 8-bit integer in 2’s complement form has a range of ______ through ______ .(10%)

- Convert -1,023ten into a 32-bit two’s complement binary number. (10%)

- Please translate the C code to assembly code in MIPS.  Assume that the variables g、h、i、j、f are assigned to $a0、$a1、$a2、$a3 and $s0. Use stack pointer($sp) and return address ($ra). (20%)

int leaf_example(int g, int h, int i, int j)

{

int f ;

f = (g + h) – (i + j) ;

return f ;

}

- The following program tries to copy words from the address in register $a0 to the address in register $a1, counting the number of words copied in register $v0. The program stops copying when it finds a word equal to 0. You do not have to preserve the contents of registers $v1, $a0, and $a1. This terminating word should be copied but not counted.

addi	$v0,	$zero,	0	# Initialize count

Loop:	lw	$v1,	0($a0)	# Read next word form source

sw	$v1,	0($a1)	# Write to destination

addi	$a0,	$a0	4	# Advance pointer to next source

addi	$a1,	$a1,	4	# Advance pointer to next destination

beq	$v1,	$zero,	loop	# Loop if word copied != zero

There are three bugs in this MIPS assembly sprogram; fix them and then write the corrected MIPS assembly program in your answer sheet. (30%)