Chapter Exam

Chapter 3 Arithmetic for Computers

2013/04/30

(10 Points)

In the IEEE-754 32-bit floating point format, why the exponent field [30-23] is placed in the higher significant bits than the fraction field [22-0]?

- The following table shows pairs of decimal numbers.

|  | A | B |
| --- | --- | --- |
| a | 69 | 90 |
| b | 102 | 44 |

(10 Points)

Assume A and B are unsigned 8-bit decimal integers. Calculate A - B. Is there overflow, underflow, or neither? why?

(10 Points)

Assume A and B are signed 8-bit decimal integers stored in sign-magnitude format. Calculate A - B. Is there overflow, underflow, or neither? why?

- Using a 4-bit version of the multiply algorithm t, let’s calculate 2ten * 3ten

(48 Points)

Complete the contents of each register on each step list on the following table.

| Step | Action | Multiplier | Multiplicand | Product |
| --- | --- | --- | --- | --- |
| 0 | Initial values | 0011 | 0000 0010 | 0000 0000 |
| 1 | Prod = Prod + Mcand |  |  |  |
|  | Lshift Mcand |  |  |  |
|  | Rshift Mplier | (1) | (5) | (9) |
| 2 | Prod = Prod + Mcand |  |  |  |
|  | Lshift Mcand |  |  |  |
|  | Rshift Mplier | (2) | (6) | (10) |
| 3 | No operation |  |  |  |
|  | Lshift Mcand |  |  |  |
|  | Rshift Mplier | (3) | (7) | (11) |
| 4 | No operation |  |  |  |
|  | Lshift Mcand |  |  |  |
|  | Rshift Mplier | (4) | (8) | (12) |

(12 Points)

Show the IEEE 754 binary representation of the number -0.75ten in single and double precision.

(20 Points)

According to the hardware, please complete the flow chart in division algorithm

(2)

Start

Test Remainder

Done

(3)

(4)

33rd repetition?

(1)

Remainder≧0

Remainder<0