Chapter Exam

Chapter 4-The Processor

2013/06/04

- Please answer the following questions.

- What’s challenge in implementation of single cycle to pipeline? (5%)

- Pipelining can decreases the execution latency of instructions. (True or False) (5%)

- Problems in this exercise assume that individual stages of the datapath have the following latencies:

| IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- |
| 300ps | 400ps | 350ps | 500ps | 100ps |

- What is the clock cycle time in a pipelined and nonpipelined processor? (10%)

- What is the total latency of a lw instruction in a pipelined ans nonpipelined processor? (10%)

- Problems in this exercise refer to the following instruction sequences:

| Instruction sequence |  |
| --- | --- |
| add sw lw add | $1, $2, $3 $2, 0 ($1) $1, 4 ($2) $2, $2, $1 |

- Find all data dependences in this instruction sequence. (Hint: data dependences involve RAW, WAR, WAW)(10%)

- Find all hazards in this instruction sequence for a five-stage pipeline with forwarding and  without forwarding. (10%)

- The following series of branch outcomes occurs for a single branch in a program. (T means the branch is taken, N means the branch is not taken.)

Branch outcomes

T, T, T, N, N,T,T,T

How many instances of this branch instruction are mis-predicted with a 1-bit and 2-bit local branch predictor, respectively? Assume that the BHT are initialized to the N state. You may assume that this is the only branch in the program. (20%)

1-bit predictor:＿＿＿＿ ;   2-bit predictor:＿＿＿＿;

- Consider executing the following code on the pipeline in Fig. 1:

| lw sub add and or | $2, 10($1) $4, $3, $2 $5, $6, $7 $8, $4, $5 $9, $8, $3 |
| --- | --- |

- At the end of the fifth cycle of execution, what instruction will be in each of the MIPS pipeline stages (IF, ID, EXE, MEM, WB)? (10%)

- If the forwarding unit is not available, what instruction will be in each of the pipeline stages at the end of the fifth cycle for this instruction sequence to be executed correctly? (10%)

- For the pipeline in Fig1, please complete following conditions, [1]~[4]? ( 20%)

Fig1: the MIPS 5-stage (IF/ID/EX/MEM/WB) pipeline with the hazard detection unit and the forwarding unit

Forwarding conditions of EX hazard

if (EX/MEM.RegWrite and (EX/MEM.RegisterRd ≠ 0)
    and (EX/MEM.RegisterRd = [1]))	ForwardA = 10

if (EX/MEM.RegWrite and (EX/MEM.RegisterRd ≠ 0)
    and (EX/MEM.RegisterRd = [2]))	ForwardB = 10

Forwarding conditions of MEM hazard

if (MEM/WB.RegWrite and (MEM/WB.RegisterRd ≠ 0)
    and (MEM/WB.RegisterRd = [3]))	ForwardA = 01

if (MEM/WB.RegWrite and (MEM/WB.RegisterRd ≠ 0)
    and (MEM/WB.RegisterRd = [4]))	ForwardB = 01