Chapter Exam

Chapter 4-1 ~ 4-4 The Processor

2013/05/14

- Please draw the single cycle datapath of load instruction. (20%)

- Please draw the single cycle datapath of add instruction (20%)

- Given the datapath of a single-cycle computer and the definition and formats of its instructions.

Complete the following table for the setting of the control lines of each instruction. Assume that the control signal Branch and the Zero output of the ALU are ANDed together to become the control.

| Instruction | RegDst | ALUSrc | Memto-Reg | RegWrite | MemRead | MemWrite | Branch | ALUOp1 | ALUOp2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add |  |  |  |  |  |  |  |  |  |
| lw |  |  |  |  |  |  |  |  |  |
| beg |  |  |  |  |  |  |  |  |  |

(1:asserted of control line; 0:deasserted of control line; x: don’t care) (20%)

Problem in this exercise refer to a clock cycle in which the processor fetches the following instruction word :

|  | Instruction word |
| --- | --- |
| a. | 1000 1100 0100 0011 0000 0000 0001 0000 |
| b. | 0001 0000 0010 0011 0000 0000 0000 1100 |

- What are the outputs of the sign-extend and the jump ”Shift left 2” unit for this instruction word ? (20%)

- What are the values of ALU control unit’s inputs for this instruction? (10%)

- Assume the following latencies for logic blocks in the datapath:

|  | I-Mem | Add | Mux | ALU | Regs | D-Mem | Sign-extend | Shift-left-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a. | 400ps | 100ps | 30ps | 120ps | 200ps | 350ps | 20ps | 0ps |
| b. | 500ps | 150ps | 100ps | 180ps | 220ps | 1000ps | 90ps | 20ps |

- What is the clock cycle time if the only type of instructions we need to support are ALU instructions (add, and, etc.)? (10%)

- What is the clock cycle time if we only had to support lw instruction? (10%)