Chapter Exam

Chapter 5.1~5.4-Large and Fast: Exploiting Memory Hierarchy

2013/06/18

- The basic concept of cache take advantage of temporal and spatial locality in the execution of application program. Show a short segment of application code in C that will have temporal and spatial locality. (10%)

- Please explain the difference between write-through and write-back and the advantage and disadvantage. (20%)

- A computer system has 32 address line and 16K byte cache. Each cache block size is 32byte. For the following case, how many tag-bit is required for each cache block?

  - A direct mapped cache (5%)

  - A full associative cache (5%)

  - A 4-way set associative cache (5%)

- The following table shows the information for L1 caches attached to each of two processors P1 and P2. Assume the memory access take 70ns and that memory access are 36% of all instructions.

|  | L1 size | L1 miss rate | L1 hit time |
| --- | --- | --- | --- |
| P1 | 1KB | 11.4% | 0.62ns |
| P2 | 2KB | 8.0% | 0.66ns |

Please calculate the AMAT (average memory access time) for each of P1 and P2? (10%)

- What is a virtual memory? (5%)

In a virtual memory system, we usually use the TLB to make address translation fast. Please describe how does a TLB operate? (10%)

- Consider a memory hierarchy using one of the three organizations for main memory show in following figure. Assume that the cache block size is 16 words, that the width of organization (b) of the figure is four words, and that the number of banks in organization (c) is four. If the main memory latency for a new access is 10 cycles and the transfer time is 1 cycles, what are the miss penalties (Time for transfer data from memory to cache) for each of these organizations (15%)

- One-word –wide memory organization

(b.) Wide memory organization

(c.) Interleaved memory organization

- Consider a paging system in which main memory has capacity of three pages. The execution of a program Q requires reference to several distinct pages as for 2 3 2 4 1 2 1 3.

The following figure is a example for FIFO(first-in first-out)：

Please finish the action of LRU (least-recently used) replacement police as bellow and indicate “HIT” in the following figure and also calculate hit rate (15%)

- The following figure shows the cache information. Assume the base CPI (ideal cache) is 2, and load & store are 36% of all instructions, please calculate the actual CPI. (Hint：Miss cycles per instruction)

(10%)

| Cache | Miss rate | Miss penalty |
| --- | --- | --- |
| Instruction cache | 2% | 100 cycles |
| Data cache | 4% | 100 cycles |